Code used for following the path with no sensor:

path= sim.getObjectHandle('Path')

box= sim.getObjectHandle('base')

joint1 = sim.getObjectHandle('Revolute1') --Programming the speed of a joint

sim.setJointTargetVelocity(joint1,0.1)

joint2 = sim.getObjectHandle('Revolute2')

sim.setJointTargetVelocity(joint1,0.1)

joint3 = sim.getObjectHandle('Revolute3')

sim.setJointTargetVelocity(joint1,0.1)

joint4 = sim.getObjectHandle('Revolute4')

sim.setJointTargetVelocity(joint1,0.1)

pathLength = sim.getPathLength(path)

posOnPath=0

v=0.01

while true do

l=posOnPath/pathLength

if(l>pathLength) then

l=pathLength

end

position=sim.getPositionOnPath(path,l)

orientation= sim.getOrientationOnPath(path,l)

-- position[3] = 0.05

sim.addStatusbarMessage(position[3])

sim.setObjectPosition(box,-1,position)

sim.setObjectOrientation(box,-1,orientation)

posOnPath=posOnPath+v*sim.getSimulationTimeStep()

sim.switchThread()

end

Code used with sonar and vision sensor:

The value of the vision in V Rep:

White ~= 0.9

Black ~= 0.1

Grey ~= 0.5

On the edge of the line I will have a value of 0.5.

function sysCall_init()

-- do some initialization here

joint1 = sim.getObjectHandle('wheel1joint')

joint2 = sim.getObjectHandle('wheel22joint')

joint3 = sim.getObjectHandle('wheel33joint')

joint4 = sim.getObjectHandle('wheel66joint')

sim.setJointTargetVelocity(joint1,0.1)

sim.setJointTargetVelocity(joint2,0.1)

sim.setJointTargetVelocity(joint3,0.1)

sim.setJointTargetVelocity(joint4,0.1)

sim.setJointTargetVelocity(jointbrush5,0.1)

sim.setJointTargetVelocity(jointbrush6,0.1)

sonar=sim.getObjectHandle('sonar')

max_dist=1

vision_sensor= sim.getObjectHandle('vision')

lineColour=0

b=0.0565 -- separation of the wheels with respect to the center of the robot not sure if it’s correct value

grey= 0.5

end

function sysCall_actuation()

-- put your actuation code here

end

function sysCall_sensing()

-- put your sensing code here

--Get distance

dist=getDistance(sonar,max_dist)

lineColour=getLineColour()

end

function sysCall_cleanup()

-- do some clean-up here

end

function getDistance(sensor)

local detected, distance

detected,distance= sim.readProximitySensor(sonar)

if(detected<1) then

distance = max_dist

end

return distance

end

function getLineColour()

value=sim.getVisionSensorImage(vision + sim.handleflag_greyscale)

return value[1]

end

while true do

result,data=sim.readVisionSensor(vision)

function followLine(lineColour,grey,k)

local wL,wR

vL1= 0.1

vL2= 0.1

vR1=0.1

vR2=0.1

return wL,wR

if(lineColour > 0.7 & lineColour < 1)

{

--keep moving

}

end

end

k- given gain

Linear speed between 0.1 and 0.5 and adjust the gain to the line to see if the line is behaiving properly